Modul 4: 	Troubleshooting in building systems and building installations: The customer reports a cold room.

Components of a heating system, task:

- Name the components shown in positions 1 to 13 in the heating diagram.

- Briefly describe the function of the components.

| Pos. | Name | Task |
| --- | --- | --- |
| 1 | Outdoor sensor | Measurement of the outside temperature |
| 2 | Room sensor | Measurement of the room temperature |
| 3 | Romm controller | Combination controller/operating unit; regulates the flow temperature of the heat generator depending on the outdoor and room sensor |
| 4 | Gas connection heat generator | Fuel supply for the heat generator |
| 5 | Power connection Heat generator | Supply of electrical energy for the heat generator |
| 6 | Pipe flow | Pipes for transporting the hot water to the radiators |
| 7 | Return flow | Pipes for transporting the cold water back to the heat generator |
| 8 | Heating pump | Ensures the transport of water in the heating circuit |
| 9 | Membrane expansion vessel | Ausgleich temperaturbedingter Volumenschwankungen im Heizungskreislauf |
| 10 | Vent valve Radiator | Equalisation of temperature-related volume fluctuations in the heating circuit |
| 11 | Radiator | Release of thermal energy to the room air mainly by convection |
| 12 | Thermostat valve | Regulates the volume flow of hot water through the radiator and thus the amount of heat energy emitted. |
| 13 | Radiator | Release of thermal energy to the room air mainly by convection |